PRODUCT DATA SHEET

# TECHNOFLUID TRANSMISSION CF4 SAE 30

## Product Description

## TECHNOFLUID Transmission CF-4 SAE 30 is a premium heavy-duty transmission and drivetrain lubricant formulated from highly refined mineral base oils and advanced additive technology. It is designed for powershift transmissions, torque converters, final drives, hydraulic systems, and wet brake applications used in construction, mining, earthmoving, and agricultural equipment. The product provides excellent wear protection, oxidation stability, friction control, and long service life under severe operating conditions.

## Applications

## JCB Equipment Transmissions

## Powershift Transmissions

## Torque Converters

## Final Drives

## Off-Highway Equipment

## Construction & Mining Machinery

## Loaders, Excavators & Backhoe Loaders

## Agricultural Machinery

## Wet Brake Systems

## Performance Level / Specifications Meets / Suitable for:

- API CF-4

- Allison C-4

- Caterpillar TO-2

- Suitable for applications requiring SAE 30 Transmission Fluid

## Typical Technical Characteristics

| Characteristics | Test Method | Typical Value |
| --- | --- | --- |
| Appearance | Visual | Clear & Bright |
| Colour | ASTM D1500 | Red |
| Density @ 29.5°C (g/cm³) | ASTM D4052 | 0.870 – 0.890 |
| Kinematic Viscosity @ 40°C (cSt) | ASTM D445 | 80 – 100 |
| Kinematic Viscosity @ 100°C (cSt) | ASTM D445 | 9 – 12 |
| Viscosity Index | ASTM D2270 | ≥ 95 |
| Flash Point (COC), °C | ASTM D92 | 210 |
| Pour Point, °C | ASTM D97 | ≤ -6 |
| Copper Corrosion (3h @100°C) | ASTM D130 | 1a |
| Foaming Characteristics | ASTM D892 | Pass |
| Oxidation Stability | ASTM D943 | Excellent |

## Key Features & Benefits

- Excellent anti-wear protection for gears and bearings

- Superior oxidation and thermal stability

- Smooth clutch and wet brake performance

- Excellent rust and corrosion protection

- High resistance to foaming

- Extended oil drain intervals

- Good seal compatibility

- Reduced transmission noise and wear